Interface Control Document (ICD)

ELECTRICAL POWER GENERATION SYSTEM TEST RIG

16-09-2025

Centre for Air Borne Systems (CABS)

Defence Research and Development Organization

Yemalur Post, Bangalore – 560037.

| Title: Interface Control Document (ICD) for Data Link System |  |
| --- | --- |
| Document No: AEW&C-MK-2-06-YY-MM-V-XX. Version 1.0 |  |
| Prepared by | CABS DEAL |
| Reviewed by | CABS DEAL |
| Verified By PD(DL) PgmC(DL) PD(Aircraft) PD(MS) PD(SI) APgD(AEW&C MK-II) |  |
| Approved PgD(AEW&C Mk-II)-CABS OEM Aircraft /ADS |  |
| Approved for release Director (CABS) |  |

Amendments Record Sheet

| Amendment No. | Subject | Issue Date | Incorporated By |  |
| --- | --- | --- | --- | --- |
|  |  |  | Name | Date |

Table of Contents

1.	DEFINITIONS, ACRONYMS AND ABBREVIATIONS	7

1.1	Definitions	7

1.1.1	Airframe External /Skin Mounted Equipment	7

1.1.2	Underfloor Mounted Equipment	7

1.1.3	Cabin Mounted Equipment	7

1.1.4	Cockpit Mounted Equipment	7

1.1.5	Aircraft Axes	7

2.	ACRONYMS AND ABBREVIATIONS	8

3.	BIBLIOGRAPHY	9

4.	SYSTEM OVERVIEW	10

5.	SYSTEM CONFIGURATION	11

6.	MECHANICAL AND INSTALLATION DETAILS	14

6.1	Physical Interfaces of Equipments	14

6.2	Mounting Details	16

6.2.1	CBDL Antenna (Top & Bottom)	16

6.2.2	Ku-Band SATCOM Antenna	21

6.2.3	RF front End (Top and Bottom)	24

6.2.4	SATCOM Radome	26

6.2.5	CAPS (Compact Antenna Pointing System for CBDL)	35

7.	COOLING DETAILS	38

7.1	Heat Load	38

7.2	Cooling Interface	39

7.3	Ground Support Equipment Cooling	40

8.	ENVIRONMENT AND EMI/LIGHTNING REQUIREMENT CHARACTERISTICS	41

9.	SAFETY CHARACTERISTICS	41

10.	OPERATIONAL CHARACTERISTICS	41

10.1	System Mode of Operation	41

10.2	System States for various mode of operation	41

11.	MAINTENANCE AND RELIABILITY	41

12.	ANTENNA ELECTRICAL CHARATERISTIC	42

12.1	CBDL System	42

12.1.1	CBDL Omni Antenna TOP and Bottom	42

12.1.2	CBDL Compact Antenna Pointing System (CAPS)	43

12.1.3	Transmitter and Receive Characteristic of C band	43

12.1.4	Antenna Radiation pattern perturbation study	43

12.1.5	Interference study with other sensor antenna	43

12.2	SATCOM System	43

12.2.1	SATCOM Antenna	43

12.2.2	Transmitter and Receiver Characteristic of Ku band system	44

12.2.3	Antenna Radiation pattern perturbation study	44

12.2.4	Interference study with other sensor antenna	44

13.	FUNCTIONAL INTERFACES	45

13.1	Discrete Signals (Blanking/WOW/1PPS etc.)	45

13.2	Audio Signals	45

13.3	Navigation Data Interfaces	45

13.4	Digital Bus Interfaces	46

13.5	Optical Interface	46

13.6	RF interface	46

13.7	Zeroize Interfaces	48

14.	CABLING INTERFACE	50

14.1	Electrical Interface Block diagram OF RACK-7(DATALINK)	58

14.2	DATALINK INTERFACE WIRING DETAILS	59

14.3	Cable Detail and Length Requirement	85

14.3.1	RF-OVER FIBER TRANSRECEIVER UNIT (TO BE DEFINED)	87

14.4	Wiring details	89

14.5	BONDING AND GROUNDING SCHEME	89

LIST OF FIGURES

Figure 1 Aircraft Axis	7

Figure 2 Tentative Antenna Position of Data link System	10

Figure 3 CBDL System Configuration Block diagram with Omni (Top) & Omni (Bottom) Antenna	11

Figure 4 CBDL System Configuration Block diagram with Omni (Top) & CAPS (Bottom) Antenna	11

Figure 5 KBDL System Configuration Block Diagram	12

Figure 5  CBDL Top and Bottom Antenna Placement	17

Figure 6 CBDL Antenna Picture	17

Figure 7 CBDL antenna Top and Bottom Drawing-1	18

Figure 8 CBDL antenna Top and Bottom Drawing-2 (Radome is part of Antenna)	19

Figure 9 CBDL antenna Top and Bottom Drawing-3	20

Figure 10 SATCOM Antenna 2D Engineering Drawing-1	21

Figure 11 SATCOM Antenna 2D Engineering Drawing-2	22

Figure 14 Tentative Picture of RF Front End	24

Figure 15 2D Engineering drawing (Tentative) of RF Front End (Top & Bottom)	25

Figure 16 Airbus 321 aircraft with SATCOM Radome	26

Figure 18 Tentative model of Configuration -1 Radome	27

Figure 17  Construction picture of Radome	27

Figure 19 Tentative model of Configuration -1 Radome	28

Figure 20 Representation of Radome and Aircraft Interface	28

Figure 21 Configuration 1 Radome Drawing	29

Figure 22 Interface requirement for CONFIGURATION -1.	30

Figure 23 radome Drawing of SATCOM Radome -Configuration 2	32

Figure 24  interface Structure drawing for Configuration-2	33

Figure 25 Picture showing Lightning Requirment	34

Figure 27 Photograph of Compact Antenna Pointing System	36

Figure 28 Compact Antenna Pointing System Model	36

Figure 29 Compact Antenna Pointing System Interfaces	37

Figure 26 Forced air cooling requirement for RF front End Top and Bottom	39

Figure 31 AZ-Radiation pattern AT 5GHz	42

Figure 32 Elevation pattern @5GHZ	42

Figure 31 Satcom Antenna Pattern	44

LIST OF TABLES

Table-1 External Mounted LRUs	12

Table 2  Cabin and Floor Board Mounted LRUs	13

# ACRONYMS AND ABBREVIATIONS

| AC GPPU | AC Ground Power Protection Unit |
| --- | --- |
| AC GPU | AC Ground Power Unit |
| ACBTC | AC Bus Tie Contactor |
| ACPMU | AC Power Management Unit |
| APC | Aircraft Power Control |
| APMU | Aircraft Power Monitoring Unit |
| APU | Auxiliary Power Unit |
| BC | Battery Contactor |
| BMS | Battery Management System |
| BOM | Bill of Materials |
| CABS | Centre for Air Borne Systems |
| CDR | Critical Design Review |
| CEMILAC | Centre for Military Aircraft Certification |
| CMM | Component Maintenance Manual |
| CMP | Configuaration Management Plan |
| CS | Current Sensor |
| CSD | Constant Speed Drive |
| CT | Current Transformer |
| DAQCS | Data Accquisition and Control System |
| DC GPPU | DC Ground Power Protection Unit |
| DC GPU | DC Ground Power Unit |
| DCBTC | DC Bus Tie Contactor |
| DCPMU | DC Power Management Unit |
| DGAQA | Director General for Aircraft Quality Assurance |
| DRDO | Defence Research and Development Establishment |
| ECN | Engineering Change Note |
| ECP | Engineering Change Proposal |

# BIBLIOGRAPHY

| S. No. | Document Title | Document Number |
| --- | --- | --- |

# INTRODUCTION

In any aircraft development program, rigs are essential to develop to test and qualify the various systems. The electrical rig is one of them and is used to develop and test the electrical LRUs, integrate them in the rig to test, qualify and evaluate the complete electrical system before it is implemented in the actual aircraft. The electrical power system is flight critical, therefore the certification process for a new aircraft requires a complete laboratory analysis of the assembled power system operating in a rig.

  - Electrical Power Generation Systems (EPGS) Test Rig defined in this document comes under Test Rigs and Tools, Testers & Ground Equipment (TTGE) which will be used for aircraft electrical system testing and evaluation activities during its development. Later the rig can be one of the TTGE of the aircraft and plays a major role in aiding the ground crew during preparation of aircraft for service, upkeep and maintenance in an efficient way. The requirement of the TTGEs can be for any development activities or for entire life cycle of the aircraft after induction into service.

  - EPGS test rig shall be designed to test the hybrid (Hydraulic Motor Driven generators and both AC and DC generation) Electrical Power Generation and Distribution System of Aircraft. The rig shall be capable of replicating the redundancy aspects of the power generation and distribution and power flow amongst different buses.

  - The basic requirement of the EPGS test rig is that it should be capable of integrating and testing the electrical system of an aircraft which will have on generating side- one AC Generator, one DC Generator and 3 no’s of Hydraulic Motor Driven Generators (HMDGs).To develop the test Rig for this hybrid configuration, the rig  shall comprise of two independent Electric Motor-Gear Drive stands to drive AC and DC Generators (available in CABS) and one Hydraulic power pack system to drive the 3 no’s of HMDGs (To be developed). At present, CABS is having the AGEST (Aircraft Generator and Electrical System Test) Facility configured for testing of Twin Engines/Generators plus APU generator up to 120kVA rating. Hence, two nos of Motor-Gear Drive stands in AGEST Facility available at CABS can be used to test  one AC Generator and one DC Generator. Therefore, Industry partner has to develop the Hydraulic Power Pack to test the three no’s of HMDGs and also integration of Hydraulic Power Pack System with the Motor-Gear System for testing the power redundancy of electrical architecture of the aircraft having hybrid power generation.

  - AC Generator have a closed loop cooling system. To cool the generator oil, an external cooling system needs to be developed, comprising an aircraft grade heat exchanger (FCOC- Fuel Cooled Oil Cooler) and aircraft fuel as the heat dissipater , to reflect the actual aircraft scenario. The development of the Fuel circulation system for the AC generator is within the scope of the Industry Partner.

  - Once the rig is equipped and functional with these prime power sources (drive stands and Hydraulic power pack), all other aspects of a typical aircraft electrical system like power conversion, distribution, control, protection and monitoring will be integrated and tested in the rig. Industry Partner shall provide the cable harness as per aircraft standard for aircraft electrical system LRUs integration and testing and validation.

  - The rig shall have the capability of testing the Electrical System LRUs of aircraft in standalone mode and in integrated mode which includes control, monitoring and analyzing various design parameters of the electrical power generating system.

  - The rig shall be capable of testing all the Electrical LRUs together as a single system replicating the actual aircraft electrical system architecture.

  - EPGS test rig shall be capable of identification, recording and display of the fault codes that are set by the individual LRUs of aircraft.

  - All the rig parameters such as flow, pressure, speed, temperature, current, voltage, contactor status, fault indications, initiated BIT status etc. shall be brought and displayed in the central console which will be placed in control room.

  - The rig shall also have the provision for Measurement Terminal/Patch Panels for manual measurement/monitoring of test and rig parameters.

# EPGS Test Rig Architecture

# The EPGS Test Rig architecture is designed to be modular, re-configurable and of sufficient ratings so that it can be adapted to meet the requirements of testing electrical systems of different architectures and aircraft.

The architecture of EPGS test rig consists of two Motor-Gear Drive stand, one each for 50kVA(or 40kVA) IDG and 5kW DC Generator, comprising of AC Drives, AC Motors, Step up Gear-boxes, Lubrication Systems for Gear-boxes, Generator cooling system, LRU Test Racks, Terminal Boxes (Patch Panels), Hydraulic Power Pack , 5 kVA HMDG Test Stand, 400W ( or 350W) HMDG Test Stands and Data Acquisition and Control System. EPGS architecture is shown as a block diagram in Figure-1 and Figure -2 shows the detail EPGS architecture along with AGEST facility present in CABS

Figure 1:EPGS Test Rig Architecture

  - In EPGS Architecture, to test the AC and DC generators two nos of Motor-Drive stands are required. AGEST facility in CABS is already having 3nos of Motor-Drive stands to test the Generators upto 120kVA. Hence, out of these drive stands , any two drive stands can be used for establishing the EPGS Rig. Integration of these drive stand with EPGS system along with control and monitoring will be in the scope of the Industry Partner.

  - The following are the Rig sub systems which are in the scope of the Industry Partner, who has to integrate them and test it as a complete rig.

    - Hydraulic Power Pack with mounting arrangements and Instrumentations

    - Test stands for 3 nos of HMDGs

    - Mounting arrangements for Generators (Both AC & DC Generators)

    - Fuel Cooled Oil Cooler (FCOC) Systems for Generator

    - Line Contactors electrical panels for interfacing Generator output to AC Loads and DC Loads

    - AC load Banks

    - DC load Banks

    - Cable looms (along with MIL grade connectors) as per Aircraft standard for complete aircraft electrical systems from generation, conversion, distribution, control, protection and monitoring (as per drawings given by CABS)

    - Electrical Panels and Racks for LRUs

    - Data Acquisition and Control System for the EPGS Test Rig and aircraft electrical LRUs

    - Testing of Aircraft Alternators/Generators with their Generator Control Units and electrical LRUs (Aircraft LRUs -Provided by CABS)

The rig is to be designed for integrated testing of complete electrical system of an Aircraft. LRUs or the EUTs of aircraft electrical system are listed here which will be tested together as a single system in the rig to validate the complete electrical system of an aircraft. The LRUs will be provided by CABS. Data acquisition and control system of electrical LRUs from central console and cable looms along with connectors are in the scope of Industry Partner.

.

| Sr. No. | LRU Name | Nomenclature/ Part No | Qty | Supplier | Remark |
| --- | --- | --- | --- | --- | --- |
| 1. | Integrated Drive Generators (IDGs) -40KVA | 761614 | 1 | HAL | IDG will be mounted on the Motor gearbox drive stand2 in the existing agest facility It will be placed near IDG |
| 2. | Generator Control Units (GCUs) 0F 40KVA | 761610A | 1 | HAL |  |
| 3. | 5 kw dc generator | GCC5A1B | 1 | HAL | IDG will be mounted on the Motor gearbox drive stand 2 in the existing AGEST facility |
| 4. | Generator Control Units (GCUs) 0F 5kw | GRCA4B | 1 | HAL | It will be placed near the IDG. |
| 5. | Hydraulic motor driven generator-350w | 928020 | 1 | HAL | It will be placed on the HMDG test bench where the hydraulic power pack will be used to power the HMDG. |
| 6. | Controlled rectifier unit-1 | 933439 mod 1 | 1 | HAL | It will be placed near the HMDG test bench. |
| 7. | Hydraulic motor driven generator-400w | 928218 | 1 | HAL | It will be placed on the HMDG test bench where the hydraulic power pack will be used to power the HMDG |
| 8. | Controlled rectifier unit-2 | 933439 mod 1 | 1 | HAL | It will be placed near the HMDG test bench. |
| 9. | Hydraulic motor driven generator-5KVA | GCC5A1B | 1 | HAL | It will be placed on the HMDG test bench where the hydraulic power pack will be used to power the HMDG |
| 10 | Generator Control Units (GCUs) 0F 5KVA | GRCA4B | 1 | HAL |  |
| 11. | AC power management unit | 263100000 | 1 | HAL | It will be placed in the LRU rack |
| 12. | DC power management unit | 26 30 00 000 | 1 | HAL | It will be placed in the LRU rack |
| 13. | Transformer rectifier unit-1 | 260100000 | 1 | HAL | It will be placed in the LRU rack. |
| 14. | Transformer rectifier unit-2 | 260100000 | 1 | HAL | It will be placed in the LRU rack. |
| 15. | Battery-1 | 6430040-1 | 1 | HAL | It will be placed in the LRU rack. |
| 16. | Battery-2 | 6430040-1 | 1 | HAL | It will be placed in the LRU rack. |
| 17. | BHEEM |  | 1 | HAL | It will be placed in the LRU rack. |
| 18. | GPPU | 1024433 | 1 | HAL |  |

# MECHANICAL AND INSTALLATION DETAILS

## Physical Interfaces of Equipments

| S. No. | LRU Name | Part number | Weight (Kg) | Dimensions(W x H x D) (in mm) | CG location | Moment of inertia (kg-m2) | Material and stiffness properties | Mounting details reference section | Engineering Drawing Reference section |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Integrated Drive Generators (IDGs) -40KVA | 761614 | 29 kg |  | All the LRU’s placement refer to the floor plan layout. All the LRU Placements refer to floor plan layout. |  |  |  |  |
|  | Generator Control Units (GCUs) 0F 40KVA | 761610A | 2.72 kg |  |  |  |  |  |  |
|  | 5 kw dc generator | GCC5A1B | 8.8 Kg |  |  |  |  |  |  |
|  | Generator Control Units (GCUs) 0F 5kw | GRCA4B | 1 kg |  |  |  |  |  |  |
|  | Hydraulic motor driven generator-350w | 928020 | 3.35 kg |  |  |  |  |  |  |
|  | Controlled rectifier unit-1 | 933439 Mod 1 |  |  |  |  |  |  |  |
|  | Hydraulic motor driven generator-350w | 928020 | 3.35 kg |  |  |  |  |  |  |
|  | Controlled rectifier unit-2 | 933439 Mod 1 |  |  |  |  |  |  |  |
|  | Hydraulic motor driven generator-5KVA | 928014 mod 1,3,4&5263100000 | 9kg |  |  |  |  |  |  |
|  | Generator Control Units (GCUs) 0F 5kVA | 928016 mod 1&2 | 2.7 kg |  |  |  |  |  |  |
|  | AC power management unit | 263100000 | 13.8 kg |  |  |  |  |  |  |
|  | DC power management unit | 26 30 000 | 14.45 kg |  |  |  |  |  |  |
|  | Transformer rectifier unit-1 | 2601 00000 | 12.3kg |  |  |  |  |  |  |
|  | Transformer rectifier unit-2 | 2601 00000 | 12.3kg |  |  |  |  |  |  |
|  | Battery-1 | 6430040-1 | 16.6 kg |  |  |  |  |  |  |
|  | Battery-2 | 6430040-1 | 16.6 kg |  |  |  |  |  |  |
|  | BHEEM |  |  |  |  |  |  |  |  |
|  | GPPU | Gppu-1024433 | 0.6 kg |  |  |  |  |  |  |

# FUNCTIONAL INTERFACE

## Discrete Signals

| SL NO | CONNECTOR PIN NO | TYPE OF SIGNAL | DESCRIPTION | REFERENCE NAME | MNEMONIC | SIGNAL AMPLITUDE | MAX/PEAK VALUE | FROM | TO |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 1 | PL1 E-35 | DI 80 | FAULT IN MAIN IDG | LD 1 | IDG_FAULT | 0V DC- TRUE | NIL-FALSE | GCU1 | BHEEM |
| 2 | PL1 E-36 | DI 81 | FAULT IN GCU35/50KVA | LD2 | GCU_1_FAULT | 0V DC- TRUE | NIL-FALSE | GCU1 | BHEEM |
| 3 | PL1 E-37 | DI 82 | FAULT IN FEEDER/GENERATOR 30/40KVA | LD3 | ALT1_FEEDER_FAULT | 0V DC- TRUE | NIL-FALSE | GCU1 | BHEEM |
| 4 | PL1 E-38 | DI 83 | SHORTED ROTATING DIODE | LD4 | ALT_1_SRD_FAULT | 0V DC- TRUE | NIL-FALSE | GCU1 | BHEEM |
| 5 | PL1 E-39 | DI 84 | SYSTEM SHUT DOWN | LD5 | GCU2_SYS_SHUTDOWN | 0V DC- TRUE | NIL-FALSE | GCU2 | BHEEM |
| 6 | PL1 E-40 | DI 85 | BIT STATUS OK GCU2 | LD6 | GCU2_BIT_OK | 0V DC- TRUE | NIL-FALSE | GCU2 | BHEEM |
| 7 | PL1 E-41 | DI 86 | SYSTEM OK GCU3 | LD7 | GCU3_SYS_OK | 0V DC- TRUE | NIL-FALSE | GCU3 | BHEEM |
| 8 | PL1 E-42 | DI 87 | FAULT IN TRU1 | LD8 | TRU1_FAULT | 0V DC- TRUE | NIL-FALSE | DCMB | BHEEM |
| 9 | PL1 E-43 | DI 88 | TRU1 OVER TEMP | LD9 | TRU1_TEMP SIGNAL | 0V DC- TRUE | NIL-FALSE | DCMB | BHEEM |
| 10 | PL1 E-44 | DI 89 | FAULT IN TRU2 | LD10 | TRU_2_FAULT | 0V DC- TRUE | NIL-FALSE | DCMB | BHEEM |
| 11 | PL1 E-45 | DI 90 | TRU2 OVER TEMP | LD11 | TRU1_TEMP SIGNAL | 0V DC- TRUE | NIL-FALSE | DCMB | BHEEM |
| 12 | PL1 E-46 | DI 91 | AIRCRAFT BATTERY HOT | LD12 | AC_BATT_HOT | 0V DC- TRUE | NIL-FALSE | DCMB | BHEEM |
| 13 | PL1 E-47 | DI 92 | GLC1 AUX CONTACTS CLOSED | LD13 | GLC1_CLOSE | 0V DC- TRUE | NIL-FALSE | ACMB | BHEEM |
| 14 | PL1 E-48 | DI 93 | GLC2 AUX CONTACTS CLOSED | LD14 | GLC2_CLOSE | 0V DC- TRUE | NIL-FALSE | ACMB | BHEEM |
| 15 | PL1 E-49 | DI 94 | GLC3 AUC CONTACTS CLOSED | LD15 | GLC3_CLOSE | 0V DC- TRUE | NIL-FALSE | ACMB | BHEEM |
| 16 | PL1 E-50 | DI 95 | TLC1 AUX CONTACTS AUX | LD16 | TLC1_CLOSE | 0V DC- TRUE | NIL-FALSE | DCMB | BHEEM |
| 17 | PL1 E-51 | DI 96 | TLC2 AUX CONTACTS AUX | LD17 | TLC2_CLOSE | 0V DC- TRUE | NIL-FALSE | DCMB | BHEEM |
| 18 | PL1 E-52 | DI 97 | AC-BTC AUX CONTACTS CLOSED | LD18 | AC_BTC_CLOSE | 0V DC- TRUE | NIL-FALSE | ACMB | BHEEM |
| 19 | PL1 E-53 | DI 98 | AC-BTC AUX CONTACTS CLOSED | LD19 | DC_BTC_CLOSE | 0V DC- TRUE | NIL-FALSE | DCMB | BHEEM |
| 20 | PL1 E-54 | DI 99 | DC EMC AUX CONTACTS CLOSED | LD20 | DC_EMC_CLOSE | 0V DC- TRUE | NIL-FALSE | DCMB | BHEEM |
| 21 | PL1 E-55 | DI 100 | BC AUX CONTACTS CLOSED | LD21 | GLC3_CLOSE | 0V DC- TRUE | NIL-FALSE | DCMB | BHEEM |
| 22 | PL1 E-56 | DI 101 | GPC AUX CONTACTS CLOSED | LD22 | GPC_1_CLOSE | 0V DC- TRUE | NIL-FALSE | ACMB | BHEEM |
| 23 | PL1 E-57 | DI 102 | TRU2 OVER TEMP | LD11 | TRU1_TEMP SIGNAL | 0V DC- TRUE | NIL-FALSE | DCMB | BHEEM |
| 24 | PL1 E-46 | DI 91 | AIRCRAFT BATTERY HOT | LD12 | AC_BATT_HOT | 0V DC- TRUE | NIL-FALSE | DCMB | BHEEM |
| 25 | PL1 E-47 | DI 92 | GLC1 AUX CONTACTS CLOSED | LD13 | GLC1_CLOSE | 0V DC- TRUE | NIL-FALSE | ACMB | BHEEM |
| 26 | PL1 E-48 | DI 93 | GLC2 AUX CONTACTS CLOSED | LD14 | GLC2_CLOSE | 0V DC- TRUE | NIL-FALSE | ACMB | BHEEM |
| 27 | PL1 E-49 | DI 94 | GLC3 AUC CONTACTS CLOSED | LD15 | GLC3_CLOSE | 0V DC- TRUE | NIL-FALSE | ACMB | BHEEM |
| 28 | PL1 E-50 | DI 95 | TLC1 AUX CONTACTS AUX | LD16 | TLC1_CLOSE | 0V DC- TRUE | NIL-FALSE | DCMB | BHEEM |
| 29 | PL1 E-51 | DI 96 | TLC2 AUX CONTACTS AUX | LD17 | TLC2_CLOSE | 0V DC- TRUE | NIL-FALSE | DCMB | BHEEM |
| 30 | PL1 E-52 | DI 97 | AC-BTC AUX CONTACTS CLOSED | LD18 | AC_BTC_CLOSE | 0V DC- TRUE | NIL-FALSE | ACMB | BHEEM |
| 31 | PL1 E-53 | DI 98 | AC-BTC AUX CONTACTS CLOSED | LD19 | DC_BTC_CLOSE | 0V DC- TRUE | NIL-FALSE | DCMB | BHEEM |
| 32 | PL1 E-54 | DI 99 | DC EMC AUX CONTACTS CLOSED | LD20 | DC_EMC_CLOSE | 0V DC- TRUE | NIL-FALSE | DCMB | BHEEM |
| 33 | PL1 E-55 | DI 100 | BC AUX CONTACTS CLOSED | LD21 | GLC3_CLOSE | 0V DC- TRUE | NIL-FALSE | DCMB | BHEEM |
| 34 | PL1 E-44 | DI 89 | FAULT IN TRU2 | LD10 | TRU_2_FAULT | 0V DC- TRUE | NIL-FALSE | DCMB | BHEEM |
| 35 | PL1 E-45 | DI 90 | TRU2 OVER TEMP | LD11 | TRU1_TEMP SIGNAL | 0V DC- TRUE | NIL-FALSE | DCMB | BHEEM |
| 36 | PL1 E-46 | DI 91 | AIRCRAFT BATTERY HOT | LD12 | AC_BATT_HOT | 0V DC- TRUE | NIL-FALSE | DCMB | BHEEM |
| 37 | PL1 E-47 | DI 92 | GLC1 AUX CONTACTS CLOSED | LD13 | GLC1_CLOSE | 0V DC- TRUE | NIL-FALSE | ACMB | BHEEM |
| 38 | PL1 E-48 | DI 93 | GLC2 AUX CONTACTS CLOSED | LD14 | GLC2_CLOSE | 0V DC- TRUE | NIL-FALSE | ACMB | BHEEM |
| 39 | PL1 E-49 | DI 94 | GLC3 AUC CONTACTS CLOSED | LD15 | GLC3_CLOSE | 0V DC- TRUE | NIL-FALSE | ACMB | BHEEM |
| 40 | PL1 E-50 | DI 95 | TLC1 AUX CONTACTS AUX | LD16 | TLC1_CLOSE | 0V DC- TRUE | NIL-FALSE | DCMB | BHEEM |
| 41 | PL1 E-51 | DI 96 | TLC2 AUX CONTACTS AUX | LD17 | TLC2_CLOSE | 0V DC- TRUE | NIL-FALSE | DCMB | BHEEM |
| 42 | PL1 E-52 | DI 97 | AC-BTC AUX CONTACTS CLOSED | LD18 | AC_BTC_CLOSE | 0V DC- TRUE | NIL-FALSE | ACMB | BHEEM |
| 43 | PL1 E-53 | DI 98 | AC-BTC AUX CONTACTS CLOSED | LD19 | DC_BTC_CLOSE | 0V DC- TRUE | NIL-FALSE | DCMB | BHEEM |
| 44 | PL1 E-54 | DI 99 | DC EMC AUX CONTACTS CLOSED | LD20 | DC_EMC_CLOSE | 0V DC- TRUE | NIL-FALSE | DCMB | BHEEM |
| 45 | PL1 E-55 | DI 100 | BC AUX CONTACTS CLOSED | LD21 | GLC3_CLOSE | 0V DC- TRUE | NIL-FALSE | DCMB | BHEEM |
| 46 | PL1 E-56 | DI 101 | GPC AUX CONTACTS CLOSED | LD22 | GPC_AUX-CL | 0V DC- TRUE | NIL-FALSE | DCMB | BHEEM |
| 47 | PL1 E-57 | DI 102 | FAULT IN TRU1_PB | LD23 | TRU1_PB_FAULT | 0V DC- TRUE | NIL-FALSE | DCMB | BHEEM |
| 48 | PL1 E-58 | DI 103 | FAULT IN TRU2_PB | LD24 | TRU2_PB_FAULT | 0V DC- TRUE | NIL-FALSE | DCMB | BHEEM |
| 49 | PL1 E-59 | DI 104 | TRU 1 HOT SIGNAL | LD25 | TRU1_HOT | 0V DC- TRUE | NIL-FALSE | TRU1 | BHEEM |
| 50 | PL1 E-60 | DI 105 | TRU2 HOT SIGNAL | LD26 | TRU2_HOT | 0V DC- TRUE | NIL-FALSE | TRU2 | BHEEM |
| 51 | PL1 E-61 | DI 106 | AC BATTERY-1 OK | LD27 | AC_BT_1_OK | 0V DC- TRUE | NIL-FALSE | BAT-1 | BHEEM |
| 52 | PL1 E-62 | DI 107 | AC BATTERY-1 MAINTENANCE | LD28 | AC_BT_1_MT | 0V DC- TRUE | NIL-FALSE | BAT-2 | BHEEM |
| 53 | PL1 E-63 | DI 108 | AC BATTERY-1 THERMAL SWITCH | LD29 | AC_BT_1_TS | 0V DC- TRUE | NIL-FALSE |  | BHEEM |
| 54 | PL1 E-64 | DI 109 | EMDP BATTERY OK | LD30 | EMDP_BT_OK | 0V DC- TRUE | NIL-FALSE |  | BHEEM |
| 55 | PL1 E-65 | DI 110 | EMDP BATTERY-1 MAINTENANCE | LD31 | EMDP_BT1_MT | 0V DC- TRUE | NIL-FALSE |  | BHEEM |
| 56 | PL1 E-66 | DI 111 | EMDP BATTERY THERMAL SWITCH | LD32 | EMDP_BT_TS | 0V DC- TRUE | NIL-FALSE |  | BHEEM |
| 57 | PL1 E-67 | DI 112 | EMDP BATTERY HOT | LD33 | EMDP_BATT_HOT | 0V DC- TRUE | NIL-FALSE |  | BHEEM |
| 58 | PL1 E-68 | DI 113 | DC GPC CLOSE STATUS | LD 34 | DC GPC_CS | 0V DC- TRUE | NIL-FALSE |  | BHEEM |
| 59 | PL1 E-71 | DI 116 | BACK UP POWER ON | LD37 | BK_UP_PWR_ON | 28V DC- TRUE | 28V DC | GCU1 | BHEEM |
| 60 | PL1 E-72 | DI 117 | GEN-2 SELECTION | LD38 | GEN2_SEL | 28V DC- TRUE | 28V DC | COCKPIT | BHEEM |
| 61 | PL1 E-73 | DI 118 | GEN-3 SELECTION | LD39 | GEN3_SEL | 28V DC- TRUE | 28V DC | COCKPIT | BHEEM |
| 62 | PL1 E-74 | DI 119 | GROUND POWER RECEPTACLE ENGAGED | LD40 | GND_PWR_ENG | 28V DC-TRUE | 28V DC | GROUND POWER RECEPTACLE | BHEEM |
| 63 | PL1 E-75 | DI 120 | GEN 3 SUPPLY ON | LD41 | GEN3_ON | 28V DC-TRUE | 28V DC | A/C | BHEEM |
| 64 | PL1 E-76 | DI 121 | HMDG1 | LD42 | HMDG1_ON | 28V DC-TRUE | 28V DC | HMDG | BHEEM |
| 65 | PL1 E-77 | DI 122 | HMDG2 | LD43 | HMDG1_OFF | 28V DC-TRUE | 28V DC | HMDG | BHEEM |
| 66 | PL1 E-78 | DI 123 | DC GPU ENG STATUS | LD44 | DC_GPU_ENG_ST | 28V DC-TRUE | 28V DC |  | BHEEM |
| 67 | PL1 E-79 | DI 124 | AC GPU ENG STATUS | LD45 | AC_GPU_ENG_ST | 28V DC-TRUE | 28V DC |  | BHEEM |

## ANALOG SIGNALS

| SL NO | CONNECTOR PIN NO | TYPE OF SIGNAL | DESCRIPTION | REFERENCE NAME | MNEMONIC | SIGNAL AMPLITUDE | MAX/PEAK VALUE | FROM | TO |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 1 | PL1 D-31 | AI 44+ | GEN1 LUBE OIL TEMP | LA1 | IDG_LUBE_OIL_TEMP | 131.5mV TO 151.5mv | 151.5 V | GCU1 | BHEEM |
| 2 | PL1 D-32 | AI 44- | GEN1 LUBE OIL TEMP RETURN | LA1- | IDG_LUBE_OIL_TEMP | _____ | _____ | GCU1 | BHEEM |
| 3 | PL1 D-33 | AI 45+ | AC_BATT_VOLT(+) | LA2 | AC_BAT_VOLT | 0 TO 50 V | 50V | ELEC | BHEEM |
| 4 | PL1 D-34 | AI 45- | AC_BATT_VOLT(-) | LA2- | AC_BAT_VOLT |  |  | ELEC | BHEEM |
| 5 | PL1 D-35 | AI 46+ | DC_BUS_VOLT(+) | LA3 | DC VOLTAGE | 0 TO 50V | 50V | DCMB | BHEEM |
| 6 | PL1 D-46 | AI 46- | DC_BUS_VOLT(-) | LA3- | DC VOLTAQGE |  |  | DCMB | BHEEM |
| 7 | PL1 D-37 | AI 47+ | EMDP_BATT_VOLT(+) | LA4 | EMDP_BATTERY_VOLT | 0 TO 32 V | 32 V | ELEC | BHEEM |
| 8 | PL1 D-38 | AI 47- | EMDP_BATT_VOLT(-) | LA4 | EMDP_BATTERY_VOLT RET |  |  |  |  |
| 9 | PL1 D-39 | AI 48+ | AC_VOLT/FREQ | LA5 | AC_GPU_POWER | 0 TO 200 V, 200mA | 200 V | ACMB | BHEEM |
| 10 | PL1 D-40 | AI 48- | AC_VOLT/FREQ | LA5- | AC_GPU_POWER Ret |  |  | ACMB | BHEEM |
| 11 | PL1 D-41 | AI 49+ | 5 KVA OVER RATING SIGNAL | LA6 | 5_KVA_0V+ | 0 TO 10V DC | 10V | ELEC | BHEEM |
| 12 | PL1 D-42 | AI 49- | 5 KVA OVER RATING SIGNAL RETURN | LA6- | 5_KVA_0V RET |  |  |  |  |
| 13 | PL1 D-43 | AI 50+ | 5Kw_current(+) | LA7 | 5_KW_0V | 0 TO 32 V DC | 32V | DCMB | BHEEM |
| 14 | PL1 D-44 | AI 50- | 5KW_CURRENT(-) | LA7- | 5_KW_OV Ret |  |  |  |  |

## Digital Bus Interfaces

| S. No. | Signal Description | From | To | Signal/ Interface Type |
| --- | --- | --- | --- | --- |
|  | AC_PMU_RS 422_CH1_RX(EL RS 422-3) | ACPMU | APMU/BHEEM | RS-422 |
|  | AC_PMU_RS 422_CH1_RX(EL RS 422-3) | ACPMU | APMU/BHEEM | RS-422 |
|  | AC_PMU_RS 422_CH2_RX(EL RS 422-3) | ACPMU | APMU/BHEEM | RS 422 |
|  | AC_PMU_RS 422_CH2_RX(EL RS 422-3) | ACPMU | APMU/BHEEM | RS 422 |
|  | DC_PMU_RS 422_CH1_RX+ | DCPMU | APMU/BHEEM | RS-422 |
|  | DC_PMU_RS 422_CH1_RX- | DCPMU | APMU/BHEEM | RS-422 |
|  | DC_PMU_RS 422_CH2_RX+ | DCPMU | APMU/BHEEM | RS-422 |
|  | DC_PMU_RS 422_CH2_RX- | DCPMU | APMU/BHEEM | RS-422 |
| 9. | A/C_BATT_CH1_ARINC_429_RX+ | BATTERY | APMU/BHEEM | ARINC-429 |
| 10. | A/C_BATT_CH1_ARINC_429_RX- | BATTERY | APMU/BHEEM | ARINC-429 |
| 11. | A/C_BATT_CH2_ARINC_429_RX+ | BATTERY | APMU/BHEEM | ARINC-429 |
| 12. | A/C_BATT_CH2_ARINC_429_RX- | BATTERY | APMU/BHEEM | ARINC-429 |
| 13. | EMDP_BATT_CH1_ARINC_429_RX+ | BATTERY | APMU/BHEEM | ARINC-429 |
| 14. | EMDP_BATT_CH1_ARINC_429_RX- | BATTERY | APMU/BHEEM | ARINC-429 |
| 15. | EMDP_BATT_CH2_ARINC_429_RX+ | BATTERY | APMU/BHEEM | ARINC-429 |
| 16. | EMDP_BATT_CH2_ARINC_429_RX- | BATTERY | APMU/BHEEM | ARINC-429 |
| 17. | 5KW_GCU3_CH1_ARINC_429_RX+ | BATTERY | APMU/BHEEM | ARINC-429 |
| 18. | 5KW_GCU3_CH1_ARINC_429_RX- | BATTERY | APMU/BHEEM | ARINC-429 |
| 19. | 5KW_GCU3_CH2_ARINC_429_RX+ | BATTERY | APMU/BHEEM | ARINC-429 |
| 20. | 5KW_GCU3_CH2_ARINC_429_RX- | BATTERY | APMU/BHEEM | ARINC-429 |

RS-422 INTERFACE

RS 422 DATA FRAME FORMAT (ACPMU):

| WORDS NO | 1 | 2 | 3 | 4 | 5 | 6 | 7 | 8 | 9 | 10 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| DATA TYPE | START WORD | START WORD | CARD TYPE(ACPMU-DAQ) | PBIT | AC VOLTAGE IN RMS |  |  | AC FREQUENCY IN HZ |  |  |
| DATA SIZE IN BITS | 16 | 16 | 16 | 16 | 16 | 16 | 16 | 16 | 16 | 16 |
| DATA | AA55 | 55AA | 0*2222 | PBIT | ACV1 | ACV2 | ACV3 | ACFREQ1 | ACFREQ2 | ACFREQ3 |

| WORDS NO | 11 | 12 | 13 | 14 | 15 | 16 | 17 | 18 | 19 | 20 | 21 | 22 | 23 | 24 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| DATA TYPE | CURRENT SENSE OUTPUT |  |  |  |  |  |  |  |  |  |  |  |  |  |
| DATA SIZE IN BITS | 16 | 16 | 16 | 16 | 16 | 16 | 16 | 16 | 16 | 16 | 16 | 16 | 16 | 16 |
| DATA | CS1 | CS2 | CS3 | CS4 | CS5 | CS6 | CS7 | CS8 | CS9 | DI(1:6) | FLASH CHECKSUM | FRAME CHECKSUM | AA54 | 55A9 |

| Position in RS422 frame | Nomencl ature | Size (in Bits) |  |  |  |  |  |  | Data (DAQ-ACPMU) |  |  |  |  |  |  |  |  |  | Remarks |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  | Bit 16 | Bit 15 | Bit 14 | Bit 13 | Bit 12 | Bit 11 | Bit 10 | Bit 9 | Bit 8 | Bit 7 | Bit 6 | Bit 5 | Bit 4 | Bit 3 | Bit 2 | Bit 1 |  |
| Word-1 | START WORD | 16 | 1 | 0 | 1 | 0 | 1 | 0 | 1 | 0 | 0 | 1 | 0 | 1 | 0 | 1 | 0 | 1 | Hex format: 0x AA55 First byte: 0x55 (LSB) Second byte: 0x AA (MSB) |
| Word-2 | START WORD | 16 | 0 | 1 | 0 | 1 | 0 | 1 | 0 | 1 | 1 | 0 | 1 | 0 | 1 | 0 | 1 | 0 | Hex format : 0x 55AA |
| Word-3 | CARD TYPE | 16 | 0 | 0 | 1 | 0 | 0 | 0 | 1 | 0 | 0 | 0 | 1 | 0 | 0 | 0 | 1 | 0 | Hex Format : 0x2222 (for DAQ- ACPMU) |
| Word-4 | PBIT | 16 | RSVD | RSVD | RSVD | RSVD | RSVD | RSVD | RSVD | EEPROM Test | Discret e BIT Test | PS 1.2V Check | +5v1_ ref Check | PS 3.3V Check | +5v2_ ref Check | analog loop back test | 0.5 REF Check | 2.5 REF Check | Hex Format : 0x01FF (for PBIT Pass) |
|  |  |  | X | X | X | X | X | X | X | 1-Pass / 0- Fail | 1-Pass / 0- Fail | 1-Pass / 0- Fail | 1-Pass / 0- Fail | 1-Pass / 0- Fail | 1-Pass / 0- Fail | 1-Pass / 0- Fail | 1-Pass / 0- Fail | 1-Pass / 0- Fail |  |
| Word-5 | ACV1 (ESS bus Volt PhA in rms) | 16 | 0 | 1 | 1 | 0 | 0 | 0 | 1 | 0 | 0 | 0 | 1 | 0 | 0 | 0 | 1 | 0 | Conversion expression: (300 * DATA)/65535 (DATA is in Decimal Format) (For 115Vrms, DATA in Hex Format : 0x6222 and DATA in Decimal Format : 25122) Range : 0 – 200Vrms |
| Word-6 | ACV2 (ESS bus Volt PhB in rms) | 16 | 0 | 1 | 1 | 0 | 0 | 0 | 1 | 0 | 0 | 0 | 1 | 0 | 0 | 0 | 1 | 0 |  |
| Word-7 | ACV3 (ESS bus Volt PhC in rms) | 16 | 0 | 1 | 1 | 0 | 0 | 0 | 1 | 0 | 0 | 0 | 1 | 0 | 0 | 0 | 1 | 0 |  |
| Word-8 | AC FREQ1 (ESS bus Freq PhA ) | 16 | 1 | 1 | 0 | 0 | 1 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | Conversion expression: (DATA*512) / 65536 (DATA is in Decimal Format) ((For 400 Hz, DATA in Hex Format : 0xC800 and DATA in Decimal Format : 51200) Range : 0 – 400Hz |
| Word-9 | AC FREQ2 (ESS bus Freq PhB ) | 16 | 1 | 1 | 0 | 0 | 1 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 |  |
| Word-10 | AC FREQ3 (ESS bus Freq PhC) | 16 | 1 | 1 | 0 | 0 | 1 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 |  |
| Word-11 | CS1 (ACGPU PhA current) | 16 | 0 | 0 | 1 | 1 | 0 | 0 | 1 | 1 | 0 | 0 | 1 | 1 | 0 | 0 | 1 | 1 | Conversion expression: (DATA*2.5) / 65535 (DATA is in Decimal Format) ((For 100A (~0.5V), DATA in Hex Format : 0x3333 and DATA in Decimal Format : 13107) Note: Current sensor output is in Volts (1 A = 0.005 V) Range : 0 – 150A (~0.75V) |
| Word-12 | CS2 (ACGPU PhB current) | 16 | 0 | 0 | 1 | 1 | 0 | 0 | 1 | 1 | 0 | 0 | 1 | 1 | 0 | 0 | 1 | 1 |  |
| Word-13 | CS3 (ACGPU PhC current) | 16 | 0 | 0 | 1 | 1 | 0 | 0 | 1 | 1 | 0 | 0 | 1 | 1 | 0 | 0 | 1 | 1 |  |

| Position in RS422 frame | Nomencl ature | Size (in Bits) |  |  |  |  |  |  | Data (DAQ-ACPMU) |  |  |  |  |  |  |  |  |  | Remarks |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  | Bit 16 | Bit 15 | Bit 14 | Bit 13 | Bit 12 | Bit 11 | Bit 10 | Bit 9 | Bit 8 | Bit 7 | Bit 6 | Bit 5 | Bit 4 | Bit 3 | Bit 2 | Bit 1 |  |
| Word-14 | CS4 (ALT1 PhA current) | 16 | 0 | 0 | 1 | 1 | 0 | 0 | 1 | 1 | 0 | 0 | 1 | 1 | 0 | 0 | 1 | 1 | Conversion expression: (DATA*2.5) / 65535 (DATA is in Decimal Format) ((For 100A (~0.5V), DATA in Hex Format : 0x3333 and DATA in Decimal Format : 13107) Note: Current sensor output is in Volts (1 A = 0.005 V) Range : 0 – 150A (~0.75V) |
| Word-15 | CS5 (ALT1 PhB current) | 16 | 0 | 0 | 1 | 1 | 0 | 0 | 1 | 1 | 0 | 0 | 1 | 1 | 0 | 0 | 1 | 1 |  |
| Word-16 | CS6 (ALT1 PhC current) | 16 | 0 | 0 | 1 | 1 | 0 | 0 | 1 | 1 | 0 | 0 | 1 | 1 | 0 | 0 | 1 | 1 |  |
| Word-17 | CS7 (ALT2 PhA current) | 16 | 0 | 0 | 0 | 0 | 0 | 1 | 1 | 0 | 0 | 1 | 1 | 0 | 0 | 1 | 1 | 0 | Conversion expression: (DATA*2.5) / 65535 (DATA is in Decimal Format) ((For 5 A (~0.0625 V), DATA in Hex Format : 0x0666 and DATA in Decimal Format : 1638) Note: Current sensor output is in Volts (1 A = 0.0125 V) Range : 0 – 25A (~0.3125V) |
| Word-18 | CS8 (ALT2 PhB current) | 16 | 0 | 0 | 0 | 0 | 0 | 1 | 1 | 0 | 0 | 1 | 1 | 0 | 0 | 1 | 1 | 0 |  |
| Word-19 | CS9 (ALT2 PhC current) | 16 | 0 | 0 | 0 | 0 | 0 | 1 | 1 | 0 | 0 | 1 | 1 | 0 | 0 | 1 | 1 | 0 |  |
| Word-20 | Di (Discrete Inputs) | 16 | RSVD | RSVD | RSVD | RSVD | RSVD | RSVD | RSVD | RSVD | RSVD | RSVD | Spare | Spare | ACBTC_ Status | GPC2_ Status | GLC_ Status | GPC1 _Stat us | Gnd : Contactor OFF Open : Contactor ON |
|  |  |  | X | X | X | X | X | X | X | X | X | X | X | X | 0-Open / 1-Gnd | 0-Open / 1-Gnd | 0-Open / 1-Gnd | 0-Open / 1-Gnd |  |
| Word-21 | SP3 (Software Checksum) | 16 | 0 | 0 | 0 | 1 | 1 | 1 | 1 | 0 | 1 | 1 | 1 | 0 | 1 | 1 | 1 | 0 | Hex format : 0x 1EE6 |
| Word-22 | Frame Checksum | 16 | X | X | X | X | X | X | X | X | X | X | X | X | X | X | X | X | Ex-OR of words from 1 to 24 except word-22 |
| Word-23 | Stop | 16 | 1 | 0 | 1 | 0 | 1 | 0 | 1 | 0 | 0 | 1 | 0 | 1 | 0 | 1 | 0 | 0 | Hex format : 0x AA54 |
| Word-24 | Stop | 16 | 0 | 1 | 0 | 1 | 0 | 1 | 0 | 1 | 1 | 0 | 1 | 0 | 1 | 0 | 0 | 1 | Hex format : 0x 55A9 |

RS 422 DATA FRAME FORMAT(DCPMU)

| WORDS NO | 1 | 2 | 3 | 4 | 5 | 6 | 7 | 8 | 9 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Data Type | Start Word | Start Word | Card Type (DCPMU-DAQ) | P-bit | DC Voltage in Volts |  |  |  |  |
| Data Size in Bits | 16 | 16 | 16 | 16 | 16 | 16 | 16 | 16 | 16 |
| DATA | 0xAA55 | 0x55AA | 0x1111 | PBIT | DC V1 | DC V2 | DC V3 | DC V4 | DC V5 |

| WORDS NO | 10 | 11 | 12 | 13 | 14 | 15 | 16 | 17 | 18 | 19 | 20 | 21 | 22 | 23 | 24 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Data Type | Temp Sensor Input |  | Current Sense Output |  |  |  |  |  | Discrete Input | Spare |  |  | Checksum | Stop | Stop |
| Data Size in Bits | 16 | 16 | 16 | 16 | 16 | 16 | 16 | 16 | 16 | 16 | 16 | 16 | 16 | 16 | 16 |
| DATA | TS1 | TS2 | CS1 | CS2 | CS3 | CS4 | CS5 | CS6 | DI (1:12) | Flash Checksum | SP2 | SP3 | Frame Checksum | AA54 | 55A9 |

| Position in RS422 frame | Nomenclat ure | Size (in Bits) |  |  |  |  |  |  |  | Data (DAQ-DCPMU) |  |  |  |  |  |  |  |  | Remarks |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  | Bit 16 | Bit 15 | Bit 14 | Bit 13 | Bit 12 | Bit 11 | Bit 10 | Bit 9 | Bit 8 | Bit 7 | Bit 6 | Bit 5 | Bit 4 | Bit 3 | Bit 2 | Bit 1 |  |
| Word-1 | START WORD | 16 | 1 | 0 | 1 | 0 | 1 | 0 | 1 | 0 | 0 | 1 | 0 | 1 | 0 | 1 | 0 | 1 | Hex format: 0x AA55 First byte: 0x55 (LSB) Second byte: 0x AA (MSB) |
| Word-2 | START WORD | 16 | 0 | 1 | 0 | 1 | 0 | 1 | 0 | 1 | 1 | 0 | 1 | 0 | 1 | 0 | 1 | 0 | Hex format: 0x 55AA |
| Word-3 | CARD TYPE | 16 | 0 | 0 | 0 | 1 | 0 | 0 | 0 | 1 | 0 | 0 | 0 | 1 | 0 | 0 | 0 | 1 | Hex Format: 0x1111 (for DAQ-DCPMU) |
| Word-4 | PBIT | 16 | RSVD | RSVD | RSVD | RSVD | RSVD | RSVD | RSVD | RSVD | EEPROM Test | Analog Loopba ck | Discrete Loopback | +1.2 V3 Ref Check | +3.3 V2 Ref Check | +5 V1 Ref Check | 0.5 REF Check | 2.5 REF Check | Hex Format: 0x00FF (for PBIT Pass) |
|  |  |  | X | X | X | X | X | X | X | X | 1-Pass / 0- Fail | 1-Pass / 0- Fail | 1-Pass / 0- Fail | 1-Pass / 0- Fail | 1-Pass / 0- Fail | 1-Pass / 0- Fail | 1-Pass / 0- Fail | 1-Pass / 0- Fail |  |
| Word-5 | TRU1_V Voltage in Volts | 16 | 0 | 1 | 0 | 1 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | Conversion Expression (DATA*32) / 65535 (DATA is in Decimal Format) (For 10V, DATA in Hex Format: 0x5000 and DATA in Decimal Format: 20480) Range is 0 to 32 V |
| Word-6 | TRU2_V Voltage in Volts | 16 | 0 | 1 | 0 | 1 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 |  |
| Word-7 | GEN3_V Voltage in Volts | 16 | 0 | 1 | 0 | 1 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 |  |
| Word-8 | EMG_Bus Voltage in Volts | 16 | 0 | 1 | 0 | 1 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 |  |
| Word-9 | GPU_V Voltage in Volts | 16 | 0 | 1 | 0 | 1 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 |  |
| Word-10 | TRU1-Temp Voltage in Volts | 16 | 0 | 1 | 0 | 0 | 1 | 1 | 0 | 0 | 1 | 1 | 0 | 0 | 1 | 1 | 0 | 1 | Conversion Expression (DATA*5.0) / 65535 (DATA is in Decimal Format) (For 1.5V, DATA in Hex Format: 0x4CCD and DATA in Decimal Format:19661) Range is 0 to 5V |
| Word-11 | TRU2-Temp Voltage in Volts | 16 | 0 | 1 | 0 | 0 | 1 | 1 | 0 | 0 | 1 | 1 | 0 | 0 | 1 | 1 | 0 | 1 |  |

| Position in RS422 frame | Nomenclat ure | Size (in Bits) |  |  |  |  |  |  | Data (DAQ-DCPMU) |  |  |  |  |  |  |  |  |  | Remarks |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  | Bit 16 | Bit 15 | Bit 14 | Bit 13 | Bit 12 | Bit 11 | Bit 10 | Bit 9 | Bit 8 | Bit 7 | Bit 6 | Bit 5 | Bit 4 | Bit 3 | Bit 2 | Bit 1 |  |
| Word-12 | GEN3-CS Voltage in Volts | 16 | 0 | 1 | 1 | 0 | 0 | 1 | 1 | 0 | 0 | 1 | 1 | 0 | 0 | 1 | 1 | 0 | Conversion Expression (DATA*2.5) / 65535 (DATA is in Decimal Format) (For 1V, DATA in Hex Format: 0x6666 and DATA in Decimal Format: 26214) Range is 0 to 2.5V |
| Word-13 | TRU1-CS Voltage in Volts | 16 | 0 | 1 | 1 | 0 | 0 | 1 | 1 | 0 | 0 | 1 | 1 | 0 | 0 | 1 | 1 | 0 |  |
| Word-14 | TRU2-CS Voltage in Volts | 16 | 0 | 1 | 1 | 0 | 0 | 1 | 1 | 0 | 0 | 1 | 1 | 0 | 0 | 1 | 1 | 0 |  |
| Word-15 | GPU-CS Voltage in Volts | 16 | 0 | 1 | 1 | 0 | 0 | 1 | 1 | 0 | 0 | 1 | 1 | 0 | 0 | 1 | 1 | 0 |  |
| Word-16 | BATT1-CS Voltage in Volts | 16 | 0 | 1 | 1 | 0 | 0 | 1 | 1 | 0 | 0 | 1 | 1 | 0 | 0 | 1 | 1 | 0 |  |
| Word-17 | BATT2-CS Voltage in Volts | 16 | 0 | 1 | 1 | 0 | 0 | 1 | 1 | 0 | 0 | 1 | 1 | 0 | 0 | 1 | 1 | 0 |  |
| Word-18 | DI (Discrete Inputs) | 16 | RSVD | RSVD | RSVD | RSVD | Spare | Spare | Spare | TLC1_ Status | TLC2_ Status | GPC2_ Status | DC-BTC_ Status | GLC3_ Status | DC-EMC_ Status | BC_Status | DC-TP_ Status | EMDP- CC_ Status | Gnd : Contactor OFF Open : Contactor ON |
|  |  |  | X | X | X | X | X | X | X | 0-Open / 1-Gnd | 0-Open / 1-Gnd | 0-Open / 1-Gnd | 0-Open / 1-Gnd | 0-Open / 1-Gnd | 0-Open / 1-Gnd | 0-Open / 1-Gnd | 0-Open / 1-Gnd | 0-Open / 1-Gnd |  |
| Word-19 | SP1 (Software Checksum) | 16 | 1 | 0 | 1 | 1 | 1 | 1 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 1 | 1 | 1 | Hex format: 0xBC07 |
| Word-20 | SP2 | 16 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | NA |
| Word-21 | SP3 | 16 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | NA |
| Word-22 | Frame Checksum | 16 | X | X | X | X | X | X | X | X | X | X | X | X | X | X | X | X | Ex-OR of words from 1 to 24 except word-22 |
| Word-23 | Stop | 16 | 1 | 0 | 1 | 0 | 1 | 0 | 1 | 0 | 0 | 1 | 0 | 1 | 0 | 1 | 0 | 0 | Hex format: 0x AA54 |
| Word-24 | Stop | 16 | 0 | 1 | 0 | 1 | 0 | 1 | 0 | 1 | 1 | 0 | 1 | 0 | 1 | 0 | 0 | 1 | Hex format: 0x 55A9 |

# ARINC 429 - TELEMETRY

The GCU broadcasts telemetry data over a single twisted pair wire, adhering to ARINC

The data flows from GCU system to other Avionic System, and represents the only interface.

The basic information element of ARINC 429 protocol is a digital word containing 32 bits. Bit Number 32 is assigned to parity, Bit Numbers 31 and 30 to the sign/status matrix, Bit Number 29 is the most significant bit of the data field.

The GCU broadcasts telemetry only through OPERATIVE CSCI

## TRANSMISSION ORDER

The Least Significant Bit (LSB) and Least Significant Character (LSC) of each word should be transmitted first. It may be noted that the least significant bit of the word is the most significant bit of the label and that the label is transmitted ahead of the data in each case.

The digital data signal to be transmitted on ARINC 429 bus is generated by the HI-3586 chip which is connected to DSP by a serial SPI bus. The SPI to ARINC data ordering is summarized in the following table.

|  |  | BIT | s transmission order |  |  |
| --- | --- | --- | --- | --- | --- |
| SPI bus | 1 | [2..3] | [4..22] | [23..24] | [25..32] |
| Arinc429 word | 32 | [30..31] | [11..29] | [9..10] | [1..8] |
| Data type | Parity error | SSM | Data | SDI | Label |

## DATA TRANSMISSION RATE

The ARINC 429 bus data rate specified for this application is Low Speed Operation and fixed at 12.5 kb/s.

## 2.1.3 	GCU ARINC 429 WORDS STRUCTURE

The ARINC 429 data transmission element is the 32 bit long word. In the following paragraphs will be described the structure of each word managed by the GCU application.

### 2.1.3.1 TLM_SYSTEM_STATUS

The TLM_SYSTEM_STATUS word collects a summary of the global status of the equipment. The word is sent with a 5HZ frequency.

|  | Meaning | Value |
| --- | --- | --- |
| 1..8 | LABEL | 200 (octal) |
| 9..10 | SDI | 0 |
| 11 | GCU System Status | 0: NOGO 1: GO |
| 12..16 | GCU Functional State | 0: Boot: Platform Init 1: Boot: Platform PBIT 2: Boot: System Init 3: Boot: Bootloader 4: Boot: System PBIT 5: Boot: Upload 6..7: Reserved 8: Normal: Idle 9: Normal: FIR 10: Normal: Generation 11: Normal: To Crash Transition 12: Reserved 14: Normal - Generation: Pre- Regulation 15: Normal - Generation: Regulation 16: Crash 24: Unknown state |
| 17 | GCU PBIT fail | 0: FALSE 1: TRUE |
| 18 | GCU CBIT fail | 0: FALSE 1: TRUE |
| 19 | GCU At least one protection trip | 0: FALSE 1: TRUE |
| 20..29 | RESERVED | N/A |
| 30..31 | SSM | 0: Normal Operation 1: Not Computed Data 2: Functional Test 3: Failure Warning |
| 32 | Parity Error | Protocol defined |

TLM_PROTECTION_STATUS

The TLM_PROTECTION_STATUS word collects information about what protections (one or more) have tripped and are not yet cleared. The word is sent with a 5HZ frequency.

|  | Meaning | Value |
| --- | --- | --- |
| 1..8 | LABEL | 201 (octal) |
| 9..10 | SDI | 0 |
| 11 | PMG Software Overcurrent | 0: not tripped 1: tripped |
| 12 | PMG Software Undercurrent | 0: not tripped 1: tripped |
| 13 | Hardware Overvoltage | 0: not tripped 1: tripped |
| 14 | Software Undervoltage | 0: not tripped 1: tripped |
| 15 | Software Overvoltage | 0: not tripped 1: tripped |
| 16 | Software Overcurrent | 0: not tripped 1: tripped |
| 17 | Reverse Current | 0: not tripped 1: tripped |
| 18 | Feeder Fault Differential Current | 0: not tripped 1: tripped |
| 19 | Rotating rectifier short diodes | 0: not tripped 1: tripped |
| 20 | Safety critical analog In PORs | 0: not tripped 1: tripped |
| 21 | Safety critical analog In SHUNTs | 0: not tripped 1: tripped |
| 22 | Underspeed | 0: not tripped 1: tripped |
| 23 | RM Over temperature Fail | 0: not tripped 1: tripped |
| 24 | GCU Over temperature Warning | 0: not tripped 1: tripped |
| 25 | GCU Over temperature Fail | 0: not tripped 1: tripped |
| 26 | PMG Phase Warning | 0: not tripped 1: tripped |
| 27 | PMG Phase Fail | 0: not tripped 1: tripped |
| 28 | RESERVED | 0: not tripped 1: tripped |
| 29 | Power Good Protection fail | 0: failed 1: passed |
| 30..31 | SSM | 0: Normal Operation 1: Not Computed Data 2: Functional Test 3: Failure Warning |
| 32 | Parity Error | Protocol defined |

### TLM_PBIT_STATUS

The TLM_PBIT_STATUS word collects the first group of information about what Power-up Built-In Tests (one or more) have failed. The word is sent with a 5HZ frequency

|  | Meaning | Value |
| --- | --- | --- |
| 1..8 | LABEL | 202 (octal) |
| 9..10 | SDI | 0 |
| 11 | GCU RAM PBIT fail | 0: failed 1: passed |
| 12 | GCU Timers PBIT fail | 0: failed 1: passed |
| 13 | GCU CPU PBIT fail | 0: failed 1: passed |
| 14 | GCU NVM PBIT fail | 0: failed 1: passed |
| 15 | GCU Watchdog PBIT fail | 0: failed 1: passed |
| 16 | GCU Safety Critical Analog PBIT fail | 0: failed 1: passed |
| 17 | GCU EOC CRC PBIT fail | 0: failed 1: passed |
| 18 | GCU Stored Data CRC PBIT fail | 0: failed 1: passed |
| 19 | GCU ARINC 429 PBIT fail | 0: failed 1: passed |
| 20 | GCU HV Generation Over-Current PBIT fail | 0: failed 1: passed |
| 21 | GCU HV Generation Over-Voltage PBIT fail | 0: failed 1: passed |
| 22 | GCU PMG Over-Current PBIT fail | 0: failed 1: passed |
| 23 | GCU Safety Critical Discrete IN PBIT fail | 0: failed 1: passed |
| 24 | GCU Safety Critical Discrete OUT PBIT fail | 0: failed 1: passed |
| 25 | GCU Thermocouple PBIT fail | 0: failed 1: passed |
| 26 | GCU Open Diodes PBIT fail | 0: failed 1: passed |
| 27 | Maintenance Bootloader PBIT fail | 0: failed 1: passed |
| 28 | Power Good PBIT fail | 0: failed 1: passed |
| 29 | RESERVED | 0: failed 1: passed |
| 30..31 | SSM | 0: Normal Operation 1: Not Computed Data 2: Functional Test 3: Failure Warning |
| 32 | Parity Error | Protocol defined |

### TLM_CBIT_STATUS

The TLM_CBIT_STATUS word collects the first group of information about what Continuous BuiltIn Tests (one or more) have failed and are not yet cleared. The word is sent with a 5HZ frequency.

|  | Meaning | Value |
| --- | --- | --- |
| 1..8 | LABEL | 203 (octal) |
| 9..10 | SDI | 0 |
| 11 | GCU Missing Clock CBIT fail | 0: failed 1: passed |
| 12 | GCU Safety Critical Discrete OUT CBIT fail | 0: failed 1: passed |
| 13 | GCU Thermocouple Converter CBIT fail | 0: failed 1: passed |
| 14 | GCU PMG Rectifier Open Diode CBIT fail | 0: failed 1: passed |
| 15 | GCU Main Rectifier Open Diode CBIT fail | 0: failed 1: passed |
| 16 | GCU 28 VDC Backup power CBIT fail | 0: failed 1: passed |
| 17..18 | RESERVED | 0 |
| 19 | GCU ARINC 429 CBIT fail | 0: failed 1: passed |
| 20..29 | RESERVED | 0 |
| 30..31 | SSM | 0: Normal Operation 1: Not Computed Data 2: Functional Test 3: Failure Warning |
| 32 | Parity Error | Protocol defined |

### TLM_TIMESTAMP

The TLM_TIMESTAMP word collects the information related to the Elapsed Time counter. The word is sent with a 5HZ frequency.

|  | Meaning | Value |
| --- | --- | --- |
| 1..8 | LABEL | 204 (octal) |
| 9..10 | SDI | 0 |
| 11..28 | measured quantity (LSB =1 sec) | 0..262143 |
| 29 | RESERVED | 0 |
| 30..31 | SSM | SSM BNR Numeric data 0: Failure_Warning 1: Not_Computed_Data (NOT USED) 2: Fuctional_Test (NOT USED) 3: Normal_Operation |
| 32 | Parity Error | Protocol defined |

### TLM_EXC_CURR

The TLM_EXC_CURR word collects the information related to the Exciter Current. The word is sent with a 5HZ frequency.

|  | Meaning | Value |
| --- | --- | --- |
| 1..8 | LABEL | 205 (octal) |
| 9..10 | SDI | 0 |
| 11..18 | zero padding | 0x00 |
| 19..29 | measured quantity LSB (weight 0.01) .. MSB (weight 1320,96) | -10.24:10.23A Resolution 0.01A |
| 30..31 | SSM | SSM BNR Numeric data 0: Failure_Warning 1: Not_Computed_Data (NOT USED) 2: Fuctional_Test (NOT USED) 3: Normal_Operation |
| 32 | Parity Error | Protocol defined |

### TLM_GEN_SHUNT_CURR

The TLM_GEN_SHUNT_CURR word collects the information related to the SHUNT 1 current. The word is sent with a 5HZ frequency.

|  | Meaning | Value |
| --- | --- | --- |
| 1..8 | LABEL | 206 (octal) |
| 9..10 | SDI | 0 |
| 11..19 | zero padding | 0x00 |
| 20..29 | measured quantity LSB (weight 0.25) .. MSB (weight 128) | 0:256A Resolution 0,25A |
| 30..31 | SSM | SSM BNR Numeric data 0: Failure_Warning 1: Not_Computed_Data (NOT USED) 2: Fuctional_Test (NOT USED) 3: Normal_Operation |
| 32 | Parity Error | Protocol defined |

### TLM_GEN_POR_VOLT

The TLM_GEN_POR_VOLT word collects the information related to the POR 1 Bus Voltage. The word is sent with a 5HZ frequency.

|  | Meaning | Value |
| --- | --- | --- |
| 1..8 | LABEL | 207 (octal) |
| 9..10 | SDI | 0 |
| 11..20 | zero padding | 0x00 |
| 21..29 | measured quantity LSB (weight 1) .. MSB (weight 256) | 0:511V Resolution 1V |
| 30..31 | SSM | SSM BNR Numeric data 0: Failure_Warning 1: Not_Computed_Data (NOT USED) 2: Fuctional_Test (NOT USED) 3: Normal_Operation |
| 32 | Parity Error | Protocol defined |

### TLM_DISCRETE_INPUT

The TLM_DISCRETE_INPUT word collects the information related to the equipment discrete signal inputs. The word is sent with a 5HZ frequency.

|  | Meaning | Value |
| --- | --- | --- |
| 1..8 | LABEL | 210 (octal) |
| 9..10 | SDI | 0 |
| 11 | GCU Power Good | 0: not active 1: active |
| 12 | Over Current Echo | 0: not active 1: active |
| 13 | Thermocouple Fault | 0: not active 1: active |
| 14 | GCU ON/OFF | 0: not active 1: active |
| 15 | GCU Over Voltage Echo | 0: not active 1: active |
| 16 | GCU DSP Reset | 0: not active 1: active |
| 17 | GCU DRDY Thermocouple | 0: not active 1: active |
| 18..29 | RESERVED | N/A |
| 30..31 | SSM | 0: Normal_Operation 1: Not_Computed_Data (NOT USED) 2: Fuctional_Test (NOT USED) 3: Failure_Warning (NOT USED) |
| 32 | Parity Error | Protocol defined |

### TLM_DISCRETE_OUTPUT

The TLM_DISCRETE_OUTPUT word collects the information related to the equipment discrete signal outputs. The word is sent with a 5HZ frequency.

|  | Meaning | Value |
| --- | --- | --- |
| 1..8 | LABEL | 211 (octal) |
| 9..10 | SDI | 0 |
| 11 | GCU Generation Status | 0: not active 1: active |
| 12 | GCU Fault | 0: not active 1: active |
| 13..29 | RESERVED | 0: not active 1: active |
| 30..31 | SSM | 0: Normal_Operation 1: Not_Computed_Data (NOT USED) 2: Fuctional_Test (NOT USED) 3: Failure_Warning (NOT USED) |
| 32 | Parity Error | Protocol defined |

### TLM_SPEED

The TLM_SPEED word collects the information related to the PMG revolutions per minute. The word is sent with a 5HZ frequency.

|  | Meaning | Value |
| --- | --- | --- |
| 1..8 | LABEL | 212 (octal) |
| 9..10 | SDI | 0 |
| 11..28 | measured quantity LSB (weight 1) .. MSB (weight 131072) | 0: 262143 RPM Resolution 1 RPM |
| 30..31 | SSM | SSM BNR Numeric data 0: Failure_Warning 1: Not_Computed_Data (NOT USED) 2: Fuctional_Test (NOT USED) 3: Normal_Operation |
| 32 | Parity Error | Protocol defined |

### TLM_TEMPERATURE

The TLM_TEMPERATURE word collects the information related to the measured GCU temperature. The word is sent with a 5HZ frequency

|  | Meaning | Value |
| --- | --- | --- |
| 1..8 | LABEL | 207 (octal) |
| 9..10 | SDI | 0 |
| 11..20 | zero padding | 0x00 |
| 21..29 | measured quantity LSB (weight 1) .. MSB (weight 256) | -256:255°C Resolution 1°C |
| 30..31 | SSM | SSM BNR Numeric data 0: Failure_Warning 1: Not_Computed_Data (NOT USED) 2: Fuctional_Test (NOT USED) 3: Normal_Operation |
| 32 | Parity Error | Protocol defined |

The software shall transmit the following data over a Low Speed ARINC 429 serial interface for 40Ah batteries:

|  |  | ARINC 429 Label Definition |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Label | Parameter Name | Data Type | Range | Units | Sig Bits | Pos Sense | Resolution | Min Transmit Interval (ms) | Max Transmit Interval (ms) |
| 271 | Battery Discrete Outputs | DISCRETE | - | - | - | - | - | 250 | 500 |
| 340 | DC Current | BNR | ± 4096 | A | 15 | Discharge | 0.125 | 250 | 500 |
| 341 | DC Voltage | BNR | 64 | V | 15 | - | 0.001953125 | 250 | 500 |
| 342 | Battery Temperature | BNR | ± 512 | C | 12 | - | 0.125 | 1000 | 2000 |
| 343 | Deliverable Energy | BNR | 512 | Ah | 13 | - | 0.0625 | 1000 | 2000 |
| 344 | Capacity | BNR | 512 | Ah | 13 | - | 0.0625 | 1000 | 2000 |
| 345 | State of Charge | BNR | 128 | % | 11 | - | 0.0625 | 1000 | 2000 |
| 346 | Estimated Life Remaining | BNR | 128 | % | 11 | - | 0.0625 | 1000 | 2000 |
| 350 | Software Version | BCD | X.YY.ZZ | - | - | - | - | 1000 | 2000 |
| 351 | Operational Status | DISCRETE | - | - | - | - | - | 500 | 1000 |
| 352 | Fault Status | DISCRETE | - | - | - | - | - | 500 | 1000 |
| 377 | Equipment ID | DISCRETE | - | - | - | - | - | 1000 | 2000 |

# Label 271 Battery Discrete Outputs

Info shall configure the discretes as follows:

| ARINC 429 Label 271 - Battery Discrete Outputs |  |  |  |
| --- | --- | --- | --- |
| Bit | Condition | State |  |
|  |  | 0 | 1 |
| 11 | OK | Not OK | OK |
| 12 | ENGINE START AVAILABLE | Available | Not Available |
| 13 | HEATING | Off | On |
| 14 | SOC GOOD | Good | Not Good |
| 15 | FAULT | No Fault | Fault |
| 16 | BATTERY HEATER DISABLE | Enabled | Disabled |
| 17 | SERVICE | Not Required | Required |

# Label 340 DC Current

For this example, the battery is reporting 74.375 Amps positive (discharge) current in the first example and -30.125 Amps negative (charge) current in the second example.

| ARINC 429 Label 340 - Current |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 32 | 31 | 30 | 29 | 28 | 27 | 26 | 25 | 24 | 23 | 22 | 21 | 20 | 19 | 18 | 17 | 16 | 15 | 14 | 13 | 12 | 11 | 10 | 9 | 8 | 7 | 6 | 5 | 4 | 3 | 2 | 1 |
| Parity | SSM |  | Data Field |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | SDI |  | Label |  |  |  |  |  |  |  |
|  |  |  | Sign | 2048 | 1024 | 512 | 256 | 128 | 64 | 32 | 16 | 8 | 4 | 2 | 1 | 0.5 | 0.25 | 0.125 | Padding |  |  |  |  | 0 |  |  | 4 |  |  | 3 |  |
| 1 | 1 | 1 | 0 | 0 | 0 | 0 | 0 | 0 | 1 | 0 | 0 | 1 | 0 | 1 | 0 | 0 | 1 | 1 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 1 | 1 | 1 |
| 0 | 1 | 1 | 1 | 1 | 1 | 1 | 1 | 1 | 1 | 1 | 0 | 0 | 0 | 0 | 1 | 1 | 1 | 1 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 1 | 1 | 1 |

# Label 341 DC Voltage

For this example, the battery is reporting a DC Voltage of 28.752 VDC:

| ARINC 429 Label 341 - DC Voltage |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 32 | 31 | 30 | 29 | 28 | 27 | 26 | 25 | 24 | 23 | 22 | 21 | 20 | 19 | 18 | 17 | 16 | 15 | 14 | 13 | 12 | 11 | 10 | 9 | 8 | 7 | 6 | 5 | 4 | 3 | 2 | 1 |
| Parity | SSM |  | Data Field |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | SDI |  | Label |  |  |  |  |  |  |  |
|  |  |  | Sign | 32 | 16 | 8 | 4 | 2 | 1 | 0.5 | 0.25 | 0.125 | 0.0625 | 0.0313 | 0.0156 | 0.00781 | 0.00391 | 0.00195 | Padding |  |  |  |  | 1 |  |  | 4 |  |  | 3 |  |
| 1 | 1 | 1 | 0 | 0 | 1 | 1 | 1 | 0 | 0 | 1 | 1 | 0 | 0 | 0 | 0 | 0 | 0 | 1 | 0 | 0 | 0 | 0 | 0 | 1 | 0 | 0 | 0 | 0 | 1 | 1 | 1 |

# Label 342 Battery Temperature

| ARINC 429 Label 342 - Battery Temperature (°C) |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 32 | 31 | 30 | 29 | 28 | 27 | 26 | 25 | 24 | 23 | 22 | 21 | 20 | 19 | 18 | 17 | 16 | 15 | 14 | 13 | 12 | 11 | 10 | 9 | 8 | 7 | 6 | 5 | 4 | 3 | 2 | 1 |
| Parity | SSM |  | Data Field |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | SDI |  | Label |  |  |  |  |  |  |  |
|  |  |  | Sign | 256 | 128 | 64 | 32 | 16 | 8 | 4 | 2 | 1 | 0.5 | 0.25 | 0.125 | Padding |  |  |  |  |  |  |  | 2 |  |  | 4 |  |  | 3 |  |

# Label 343 Deliverable Energy

| ARINC 429 Label 343 - Deliverable Energy (Ah) |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 32 | 31 | 30 | 29 | 28 | 27 | 26 | 25 | 24 | 23 | 22 | 21 | 20 | 19 | 18 | 17 | 16 | 15 | 14 | 13 | 12 | 11 | 10 | 9 | 8 | 7 | 6 | 5 | 4 | 3 | 2 | 1 |
| Parity | SSM |  | Data Field |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | SDI |  |  |  |  | Label |  |  |  |  |
|  |  |  | Sign | 256 | 128 | 64 | 32 | 16 | 8 | 4 | 2 | 1 | 0.5 | 0.25 | 0.125 | 0.063 | Padding |  |  |  |  |  |  | 3 |  |  | 4 |  |  | 3 |  |

# Label 344 Capacity

| ARINC 429 Label 344 - Capacity (Ah) |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 32 | 31 | 30 | 29 | 28 | 27 | 26 | 25 | 24 | 23 | 22 | 21 | 20 | 19 | 18 | 17 | 16 | 15 | 14 | 13 | 12 | 11 | 10 | 9 | 8 | 7 | 6 | 5 | 4 | 3 | 2 | 1 |
| Parity | SSM |  | Data Field |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | SDI |  |  |  |  | Label |  |  |  |  |
|  |  |  | Sign | 256 | 128 | 64 | 32 | 16 | 8 | 4 | 2 | 1 | 0.5 | 0.25 | 0.125 | 0.063 | Padding |  |  |  |  |  |  | 4 |  |  | 4 |  |  | 3 |  |

# Label 345 State of Charge

| ARINC 429 Label 345 - State of Charge (%) |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 32 | 31 | 30 | 29 | 28 | 27 | 26 | 25 | 24 | 23 | 22 | 21 | 20 | 19 | 18 | 17 | 16 | 15 | 14 | 13 | 12 | 11 | 10 | 9 | 8 | 7 | 6 | 5 | 4 | 3 | 2 | 1 |
| Parity | SSM |  | Data Field |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | SDI |  |  |  |  | Label |  |  |  |  |
|  |  |  | Sign | 64 | 32 | 16 | 8 | 4 | 2 | 1 | 0.5 | 0.25 | 0.125 | 0.063 | Padding |  |  |  |  |  |  |  |  | 5 |  |  | 4 |  |  | 3 |  |

# Label 346 Estimated Life Remaining

| ARINC 429 Label 346 -Estimated Life Remaining (%) |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 32 | 31 | 30 | 29 | 28 | 27 | 26 | 25 | 24 | 23 | 22 | 21 | 20 | 19 | 18 | 17 | 16 | 15 | 14 | 13 | 12 | 11 | 10 | 9 | 8 | 7 | 6 | 5 | 4 | 3 | 2 | 1 |
| Parity | SSM |  | Data Field |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | SDI |  |  |  |  | Label |  |  |  |  |
|  |  |  | Sign | 64 | 32 | 16 | 8 | 4 | 2 | 1 | 0.5 | 0.25 | 0.125 | 0.063 | Padding |  |  |  |  |  |  |  |  | 6 |  |  | 4 |  |  | 3 |  |

- Connector Details

  - ALT1-30/40 KVA IDG

| Sl.no | Connector ID socket Side | Function | Connector PN & backshell | Mating connector PN & Backshell |
| --- | --- | --- | --- | --- |
| 1 | J1 1Xa | monitoring & Control |  | MS27467E13B98S |
| 2 | J2 1Xb | monitoring & Control |  | MS27467E15B97S |
| 3 | T1, T2, T3 & N | 115V,400Hz- power terminal (to ACPMU-1,2,3&4) (#2AWG) |  |  |

1.2 GCU-ALT1

| Sl.no | Connector ID socket Side | Function | Connector PN & backshell | Mating connector PN & Backshell |
| --- | --- | --- | --- | --- |
| 1 | J1 2Xa | monitoring & Control |  | MS27467T23B53S |

- ACPMU

| Sl.no | Connector ID socket Side | Function | Connector PN & backshell | Mating connector PN & Backshell |
| --- | --- | --- | --- | --- |
| 1 | J1 3Xa | Phase monitoring & control | 97B4102E32B13S | 97B4106F32B13P |
| 2 | J2 3Xb | Phase and monitoring | 97B4102E32B13SX | 97B4106F32B13PX |
| 3 | J3 3Xc | Monitoring and control | D38999/20WF35SN | D38999/26WF35PN |
| 4 | J4 3Xd | Monitoring and control | 97B4102E2222S | 97B4106F2222P |
| 5 | J5 3Xe | Monitoring and control | D38999/20WJ24SN | D38999/26WJ24PN |
| 6 | TERMINAL BLOCK-1(TB-1) | AC OUTPUT FROM ACGPU |  |  |
| 7 | TERMINAL BLOCK-2(TB-2) | AC OUTPUT FOR ALT2 |  |  |

- ALT2-5KVA HMDG

| Sl.no | Connector ID socket Side | Function | Connector PN & backshell | Mating connector PN & Backshell |
| --- | --- | --- | --- | --- |
| 1 | C1 | Monitoring and control | D38999/20FE06SN | D38999/26FE06PN |
| 2 | C2 | monitoring & Control | D38999/20FC98SN | D38999/26FC98PN |
| 3 | C3 | monitoring & Control | D38999/20YA98PA | M83723/95K8986 |
| 4 | C4 | monitoring & Control | D38999/20YA98PN | D38999/26FA98SN |

  - GCU-ALT2

| Sl.no | Connector ID socket Side | Function | Connector PN & backshell | Mating connector PN & Backshell |
| --- | --- | --- | --- | --- |
| 1 | J1 5Xa | monitoring & Control | D38999/20FE26PN | D38999/26FE26SN |

- GPPU

| Sl.no | Connector ID socket Side | Function | Connector PN & backshell | Mating connector PN & Backshell |
| --- | --- | --- | --- | --- |
| 1 | 7Xa | Power backup, monitoring & Control | D38999/20FC35PN | D38999/26FC35SN |

- TRU-1

| Sl.no | Connector ID socket Side | Function | Connector PN & backshell | Mating connector PN & Backshell |
| --- | --- | --- | --- | --- |
| 1 | 6Pa | monitoring & Control | D38999/20FE06PN | D38999/26FE06SN |
| 2 | 6Pb | monitoring & Control | D38999/20FB35PN | D38999/26FB35SN |
| 3 | 1,2 | (#00AWG TRU-1 INPUT TO DCPMU) |  |  |

- TRU-2

| Sl.no | Connector ID socket Side | Function | Connector PN & backshell | Mating connector PN & Backshell |
| --- | --- | --- | --- | --- |
| 1 | 7Pa | monitoring & Control | D38999/20FE06PN | D38999/26FE06SN |
| 2 | 7Pb | monitoring & Control | D38999/20FB35PN | D38999/26FB35SN |
| 3 | 1,2 | (#00AWG TRU-2 INPUT TO DCPMU) |  |  |

- DCPMU

| Sl.no | Connector ID socket Side | Function | Connector PN & backshell | Mating connector PN & Backshell |
| --- | --- | --- | --- | --- |
| 1 | T1 T2 | (TRU-1 INPUT) Power terminal to DC GPR |  |  |
| 2 | J1 8Pa | Status, monitoring & Control | D38999/20WH54SN | D38999/26WH54PN |
| 3 | J2 8Pb | monitoring & Control | D38999/20WG35SN | D38999/26WG35PN |
| 4 | T8 T7 | (TRU-1 INPUT) Power terminals to DC generator |  |  |
| 5 | T11(+) T12(-) | Power terminals from 40 AH Li-Ion A/C BATTERY |  |  |
| 6 | T9(+) T10(-) | EMDP voltage to 14P - 40 AH Li-Ion A/C BATTERY |  |  |
| 7 | T15(EMG-3) & T16 RTN(EMG-3) | (To load bank and nominal current rating) |  |  |
| 8 | J4 8Pd | Monitoring and control (To load bank and nominal current rating) | 97B4102E2222S | 97B4106F2222P |
| 9 | T13(EMG-2) & T14RTN(EMG-2) | (To JFS and nominal current rating) |  |  |
| 10 | J3 | Monitoring and control | 97B4102E2410S | 97B4106F2410P |
| 11 | T17(MB-3) T18 RTN(MB-3) | monitoring (To load bank and nominal current rating) | 97B4102E2222SX | 97B4106F2222PX |
| 12 | J5 8Pe | Monitoring (To load bank and nominal current rating) | 97B4102E2222SX | 97B4106F2222PX |
| 13 | T4 & T3 | TRU-1 INPUT |  |  |
| 14 | T5 & T6 | TRU-2 INPUT |  |  |

- 5KW DC GENERATOR

| Sl.no | Connector ID socket Side | Function | Connector PN & backshell | Mating connector PN & Backshell |
| --- | --- | --- | --- | --- |
| 1 | J1 | Monitoring & Control | D38999/20MC98SN | D38999/26FC98PN |
| 2 | (+)(-) | Power Terminals (to DCPMU-TRU1 INPUT) |  |  |

  - GCU-DC GENERATOR

| Sl.no | Connector ID socket Side | Function | Connector PN & backshell | Mating connector PN & Backshell |
| --- | --- | --- | --- | --- |
| 1 | J1 5Pa | Monitoring & Control | CTVPS02RF-13-98PN(P35) | D38999/26MC98SN |
| 2 | J2 5Pb | Monitoring & Control | CTVPS02RF-17-26PN(P35) | D38999/26ME26SN |

- BATTERY-1

| Sl.no | Connector ID socket Side | Function | Connector PN & backshell | Mating connector PN & Backshell |
| --- | --- | --- | --- | --- |
| 1 | J1 | Power terminal to DCPMU |  | MS25182-2 |
| 2 | J2 | Monitoring and control |  | MS3126F14-18S |

- BATTERY-2

| Sl.no | Connector ID socket Side | Function | Connector PN & backshell | Mating connector PN & Backshell |
| --- | --- | --- | --- | --- |
| 1 | J1 | Power terminal to DCPMU |  | MS25182-2 |
| 2 | J2 | Monitoring and control |  | MS3126F14-18S |

- 400W HMDG-1

| Sl.no | Connector ID socket Side | Function | Connector PN & backshell | Mating connector PN & Backshell |
| --- | --- | --- | --- | --- |
| 1 | J1 | Monitoring and control | D38999/20FC08SN | D38999/26FC08PA |

- 400W HMDG-2

| Sl.no | Connector ID socket Side | Function | Connector PN & backshell | Mating connector PN & Backshell |
| --- | --- | --- | --- | --- |
| 1 | J1 | Monitoring and control | D38999/20FC08SN | D38999/26FC08PA |

- Rectifier Converter-1

| Sl.no | Connector ID socket Side | Function | Connector PN & backshell | Mating connector PN & Backshell |
| --- | --- | --- | --- | --- |
| 1 | J1 | Monitoring and control | D38999/20FE06PN | D38999/26FE06SN |
| 2 | J2 | Monitoring and control | D38999/20FD19PN | D38999/26FD19SN |