| SL NO. | LRU NAME | TYPE OF SIGNAL | SIGNAL NAME | SIGNAL RANGE |
| --- | --- | --- | --- | --- |
| 1. | GCU-ALT 1 | DISCRETE SIGNAL | INT_BIT_GCU_1 | 0/28V |
| 2. |  |  | ALT_1_FEEDER_FAULT | GND(T)/OPEN(F) |
| 3. |  |  | GCU_1_FAULT | GND(T)/OPEN(F) |
| 4. |  |  | BKUP_POWER_ON | 28 V(T)/OPEN(F) |
| 5. |  |  | ALT_1_SRD_FAULT | GND(T)/OPEN(F) |
| 6. |  |  | IDG_FAULT | GND(T)/OPEN(F) |
| 7. |  | ANALOG SIGNAL | IDG_LUBE_OIL_TEMP |  |
| 8. |  |  | GCU_BODY TEMP |  |
| 9. | ALT1-35/40KVA | ANALOG SIGNAL | IDG_VOLTAGES_A,B,C PHASES | 0-200 VRMS |
| 10. |  |  | IDG_CURRENTS_A,B,C PHASES | 0-150A |
| 11. |  |  | IDG_PHASE SEQUENCE |  |
| 12. |  |  | IDG_VIBRATION |  |
| 13. |  |  | IDG_FREQUENCY |  |
| 14. |  |  | IDG_BODY TEMP |  |
| 15. |  |  | IDG_OIL I/L AND O/L TEMP | 0-200°C |
| 16. |  |  | IDG_OILO I/L PR AND O/L PR | 0-50 BAR |
| 17. |  |  | IDG_OIL FLOW | 0-50 LPM |
| 18. |  |  | IDG_OIL SUMP TEMPERATURE |  |
| 19. |  |  | FCOC_PARAMETERS |  |
| 20. | GCU-ALT 2 | DISCRETE SIGNAL | START/STOP HMDG | PRESSURE SWITCH |
| 21. |  |  | INIT_BIT_GCU_2 | 0-28V |
| 22. |  |  | GCU_2_SYS_SHUTDOWN | GND(T)/OPEN(F) |
| 23. |  |  | GCU_2_BIT OK | GND(T)/OPEN(F) |
| 24. | ALT-2 5KVA | ANALOG SIGNAL | ALT_2 VOLTAGES A/B/C PHASES | 0-200VRMS |
| 25. |  |  | ALT_2 CURRENT A/B/C PHASES |  |
| 26. |  |  | ALT_2 PHASE SEQUENCE |  |
| 27. |  |  | ALT_2 FREQUENCY |  |
| 28. |  |  | ALT_2 HYD OIL I/L&O/L CASE DRAIN TEMP |  |
| 29. |  |  | ALT_ 2 BODY TEMP |  |
| 30. |  |  | GCU_2 BODY TEMP |  |
| 31. | ACPMU | RS 422-24 PARAMETERS | START WORD | AA55 |
|  |  |  | START WORD | 55AA |
|  |  |  | CARD TYPE | 2222 |
|  |  |  | PBIT | 01FF |
|  |  |  | ACV1/ACV2/ACV3_ESS BUS | 0-200VRMS |
|  |  |  | ACFREQ1/ACFREQ2/ACFEQ3_ESS BUS | 0-400HZ |
|  |  |  | CS1/CS2/CS3_ACGPU PHA/PHB/PHC | 0-150A |
|  |  |  | ALT1 PHA/PHB/PHC CURRENT | 0-150A |
|  |  |  | ALT2 PHA/PHB/PHC CURRENT | 0-25A |
|  |  |  | DISCRETE I/P-ACBTC/GLC1/GLC2/GPC1 | OPEN(0)/GND(1) |
|  |  |  | SP3 SOFTWARE CHECKSUM | 1EE6 |
|  |  |  | FRAME CHECKSUM | EX OR OF WORDS |
|  |  |  | STOP | AA54 |
|  |  |  | STOP | 55A9 |
| 32. | DCPMU | RS 422-24 PARAMETERS | START WORD | AA55 |
|  |  |  | START WORD | 55AA |
|  |  |  | CARD TYPE | 1111 |
|  |  |  | TRU1_V VOLTAGE | 0-32V |
|  |  |  | TRU2_V VOLTAGE | 0-32V |
|  |  |  | GEN 3_V VOLTAGE | 0-32V |
|  |  |  | EMG_BUS VOLTAGE | 0-32V |
|  |  |  | GPU_VOLTAGE V | 0-32V |
|  |  |  | TRU1_TEMP | 0-5V |
|  |  |  | TRU2_TEMP | 0-5V |
|  |  |  | GEN3_CS/TRU1_CS/TRU2_CS/GPU_CS/BATT2_CS | 0-2.5V |
|  |  |  | TLC_1/TLC_2/GPC_2/DC-BTC/GLC3/DC-EMC/BC-STATUS/EMDP-CC_STATUS/DC-TP_STATUS | OPEN(0)/GND(1) |
|  |  |  | SP3 SOFTWARE CHECKSUM | BC07 |
|  |  |  | FRAME CHECKSUM | EX OR OF WORDS |
|  |  |  | STOP | AA54 |
|  |  |  | STOP | 55A9 |
| 33. | TRU-1 | ANALOG SIGNAL | TRU-1 VOLTAGE MON TRU-1 VOLTAGE MON TRN | 0-32V |
| 34. |  |  | TRU-1 TEMP SIGNAL TRU-1 TEMP SIGNAL TRN | 0-1.5V(0-150°C) |
| 35. | TRU-2 | ANALOG SIGNAL | TRU-2 VOLTAGE MON TRU-2 VOLTAGE MON TRN | 0-32V |
| 36. |  |  | TRU-2 TEMP SIGNAL TRU-2 TEMP SIGNAL TRN | 0-1.5V(0-150°C) |
| 30. | BATT1 | ARINC-429 A(H)/B(U)-50 PARAMETERS | BATTERY DISCRETE OUTPUTS DC CURRENT BATTERY TEMPERATURE DELIVERABLE ENERGY DC_VOLTAGE CAPACITY STATE OF CHARGE ESTIMATED LIFE REMAINING SOFTWARE VERSION OPERATIONAL STATUS FAULT STATUS EQUIPMENT ID | 0-11V |
|  |  | ANALOG SIGNAL | A/C_BATT_TEMP_1 (+)D A/C_BATT_TEMP_1 (+)E A/C_BATT_TEMP_2 (+)F A/C_BATT_TEMP_2 (+)G | -40-260°C(0-10V) -40-260°C(0-10V) |
| 31. | BATT2-EMDP | ARINC-429 A(H)/B(U)-50 PARAMETERS | BATTERY DISCRETE OUTPUTS DC CURRENT BATTERY TEMPERATURE DELIVERABLE ENERGY DC_VOLTAGE CAPACITY STATE OF CHARGE ESTIMATED LIFE REMAINING SOFTWARE VERSION OPERATIONAL STATUS FAULT STATUS EQUIPMENT ID | 0-11V |
|  |  | ANALOG SIGNAL | A/C_BATT_TEMP_1 (+)D A/C_BATT_TEMP_1 (+)E A/C_BATT_TEMP_2 (+)F A/C_BATT_TEMP_2 (+)G | -40-260°C(0-10V) -40-260°C(0-10V) |
| 32. | VRU-1 | DISCRETE SIGNAL | FAULT BIT 1 | GND(T)/OPEN (F) |
|  |  |  | FAULT BIT 2 | GND(T)/OPEN (F) |
| 33. | VRU-2 | DISCRETE SIGNAL | FAULT BIT 1 | GND(T)/OPEN (F) |
|  |  |  | FAULT BIT 2 | GND(T)/OPEN (F) |
| 34. | 5KW DC GEN | ANALOG SIGNAL | GEN_VOLTAGE |  |
| 35. |  |  | GEN_CURRENT |  |
|  |  | ARINC 429 | TLM_SYSTEM STATUS | 0-11V |
|  |  |  | TLM_PROTECTION_STATUS |  |
|  |  |  | TLM_PBIT_STATUS |  |
|  |  |  | TLM_CBIT_STATUS |  |
|  |  |  | TLM_TIMESTAMP |  |
|  |  |  | TLM_EXC_CURRENT |  |
|  |  |  | TLM_GEN_SHUNT_CURRENT |  |
|  |  |  | TLM_GEN_POR_CURRENT |  |
|  |  |  | TLM_DISCRETE_INPUT |  |
|  |  |  | TLM_DISCRETE_OUTPUT |  |
|  |  |  | TLM_SPEEDTLM_TEMPERATURE |  |
| 36. | GPPU | ANALOG SIGNALS | GPU VOLTAGE_A/B/C PHASES | 0-200VRMS |
|  |  |  | GPU_CURRENTS A/B/C PHASES | 0-150A |
|  |  |  | GPU_PHASE SEQUENCE |  |
| 37. | HMDG 1 | ANALOG SIGNALS | HMDG_A/B/C VOLTAGES | 0-200VRMS |
|  |  |  | HMDG_A/B/C CURRENTS |  |
|  |  |  | HMDG_FLOW/PRESSURE |  |
|  |  |  | HMDG_O/P FREQ |  |
|  |  | DISCRETE SIGNALS | HMDG_STATUS | 28V(TRUE)/OPEN(FALSE) |
|  |  |  | BIT_OK | 28V(TRUE)/OPEN(FALSE) |
| 38. | HMDG 2 | ANALOG SIGNALS | HMDG_A/B/C VOLTAGES | 0-200VRMS |
|  |  |  | HMDG_A/B/C CURRENTS |  |
|  |  |  | HMDG_FLOW/PRESSURE |  |
|  |  |  | HMDG_O/P FREQ |  |
|  |  | DISCRETE SIGNALS | HMDG_STATUS | 28V(TRUE)/OPEN(FALSE) |
|  |  |  | BIT_OK | 28V(TRUE)/OPEN(FALSE) |